Through-hole package

1N914 & 1N4148 in DO-35 glass axial package.

Surface-mount packages

LL914 & LL4148 in MiniMELF package.

1N4148W in SOD-123 package.

Note: Some 1N4148 family surface-mount packages are marked with "T4" text.

Specifications

Absolute maximum ratings (stress ratings, consult datasheet for recommended ratings).

VRRM = 100 V  (maximum repetitive reverse voltage)

IO = 200 mA  (average rectified forward current)

IF = 300 mA  (DC forward current)

If = 400 mA  (recurring peak forward current)

IFSM = 1 A at 1 s pulse width; 4 A at 1 μs pulse width (non-repetitive peak forward surge current)

Electrical and Thermal characteristics:

VF = 1 V at 10 mA (maximum forward voltage)

VR = 75 V at 5 μA; 100 V at 100 μA (minimum breakdown voltage and reverse leakage current)

trr = 4 ns (maximum reverse-recovery time)

PD = 500 mW (maximum power dissipation